'BOLT IN' REPLACEMENT GM A-BODY CHASSIS. PROTO-TYPE PICTURED.

SUBSEQUENT CHASSIS' WOULD CONSIST OF AND INCLUDE;

2 X 4 11 GA. TUBE CONSTUCTION. FRONT AND REAR FRAME RAILS MANDREL BENT.

TUBULAR FRONT A-ARMS AND REAR CONTROL ARMS.

SET UP FOR COIL OVER SHOCKS ON ALL CORNERS.

ALL SUSPENSION PICK UP POINTS ARE TEFLON LINED HEIM JOINTS OR

MOLY-IMPREGNATED NYLON BUSHINGS.

SET UP FOR B-BODY GM SPINDLES.

FRONT AND REAR ADJUSTABLE FOR RIDE HEIGHT IN 1" INCREMENTS.

REAR CONTROL ARMS ARE ADJUSABLE FOR CHANGING INSTANT CENTER.

STEERING WILL BE C4 VETTE RACK AND PINION.

STEERING WILL BE ADJUSTABLE FOR BUMPSTEER.

SWAY BARS, PANHARD BAR, BODY MOUNT KIT WOULD BE INCLUDED.

INITIAL CHASSIS WILL BE SET UP FOR CHEVY ENGINES.

NOTE---BUILT USING YOUR SUPPLIED REAR, RACK, SPINDLES, COIL OVERS.

ANY COMMENTS OR QUESTIONS WELCOME.

THANK YOU, DOUG ANDERSON